Customer: 1000 RPM, 100% humidity, 45 deg Celsius,

Job#: Random vib

Data: C:\Users\ESSII\Desktop\Auto engine\2022May21-0617-0005.vrd

Test: C:\Users\ESSII\Desktop\auto engine .5g.vrp

Data stored on May 21, 2022 06:19:39

-  -

Running

Breakpoint table

Frequency	G²/Hz	dB/Octave

10 Hz	0.0019	0

40 Hz	0.0019	-3

500 Hz	0.0001533

Test level schedule:

Duration	Level

1)	0:00:05	25 %

2)	0:00:10	50 %

3)	0:01:00	100 %

** Test started May 21, 2022 06:17:26, running for 0:02:13

** Current level: 3, running at 100 % for 0:00:47 of 0:01:00

Measurements:

Demand: 0.5 G RMS	1.206 mm pk-pk

Control: 0.5083 G RMS	1.17 mm pk-pk

Channel Measurements:

(Overall)	(InBand)

Ch1 	0.5108 G RMS	0.501 G RMS

Ch2	0.5372 G RMS	0.5151 G RMS

Ch3	1.052 G RMS	1.051 G RMS

Ch4	0.001697 G RMS	0.001445 G RMS

Channel Calibration:

Ch1	10.11 mV/G 	110813, Mar 08, 2022

Ch2	97.2 mV/G	16662, Dec 27, 2021

Ch3	96.8 mV/G	16666, Dec 21, 2021

Ch4	10.11 mV/G	110817, Mar 08, 2022

Hardware calibration details:

Serial	Date of	Days since	Calibration	Days until

Number	last cal	last cal	due date	cal due

9536C9BC	Sep 15, 2020	612 days	Sep 15, 2021	Past due by 247 days

Test Notes:

Test notes: none